#include <dummy.h>

#include <Wire.h>

#include <DHT.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

// ---------- SAFE PIN CONFIGURATION ----------

#define DHT_PIN       4       // DHT11 Data Pin

#define DHT_TYPE      DHT11

#define FAN_PIN       5       // Safe GPIO driving BC548B Transistor Base (Avoids PSRAM)

#define I2C_SDA       8       // Custom I2C SDA for ESP32-S3

#define I2C_SCL       9       // Custom I2C SCL for ESP32-S3

// ---------- OLED DISPLAY CONFIGURATION ----------

#define SCREEN_WIDTH  128

#define SCREEN_HEIGHT 64

#define OLED_RESET    -1      // Reset pin (-1 if sharing ESP32 reset)

#define SCREEN_ADDRESS 0x3C   // Default I2C address for 0.96" OLED

// ---------- TEMPERATURE LIMITS ----------

#define FAN_ON_TEMP   35.0f   // Turn ON Fan at or above 35.0°C

#define FAN_OFF_TEMP  30.0f   // Turn OFF Fan at or below 30.0°C

// Transistor / Load State Definitions

#define FAN_ON        HIGH

#define FAN_OFF       LOW

DHT dht(DHT_PIN, DHT_TYPE);

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, OLED_RESET);

bool fanON = false;

void setup() {

Serial.begin(115200);

delay(1000); // Wait for Serial Monitor

Serial.println("\n==================================");

Serial.println("  TEMP FAN & OLED CONTROLLER      ");

Serial.println("==================================");

pinMode(FAN_PIN, OUTPUT);

digitalWrite(FAN_PIN, FAN_OFF); // Ensure Fan starts OFF

// Initialize Custom I2C Pins for ESP32-S3

Wire.begin(I2C_SDA, I2C_SCL);

// Initialize OLED Display

if (!display.begin(SSD1306_SWITCHCAPVCC, SCREEN_ADDRESS)) {

Serial.println(F("ERROR: SSD1306 OLED allocation failed! Check I2C wiring."));

for (;;); // Halt execution if display fails

}

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 10);

display.println(F(" ESP32-S3 FAN CONTROL "));

display.println(F("   Initializing...    "));

display.display();

dht.begin();

Serial.println("DHT11 Initialized on GPIO 4.");

}

void loop() {

// DHT11 requires a minimum 2-second delay between readings

delay(2000);

float temperature = dht.readTemperature();

float humidity = dht.readHumidity();

// Check for sensor read errors

if (isnan(temperature) || isnan(humidity)) {

Serial.println("ERROR: Failed to read from DHT11 sensor! Check wiring on GPIO 4.");

display.clearDisplay();

display.setCursor(0, 20);

display.setTextSize(1);

display.println(F(" SENSOR READ ERROR! "));

display.println(F("Check DHT11 Wiring"));

display.display();

return;

}

// Temperature threshold logic (Hysteresis)

if (temperature >= FAN_ON_TEMP) {

fanON = true;

} else if (temperature <= FAN_OFF_TEMP) {

fanON = false;

}

digitalWrite(FAN_PIN, fanON ? FAN_ON : FAN_OFF);

// Serial Monitor Debug Output

Serial.print("Temp: ");

Serial.print(temperature, 1);

Serial.print(" C   |   Humidity: ");

Serial.print(humidity, 1);

Serial.print(" %   |   Fan State: ");

Serial.println(fanON ? "ON" : "OFF");

// Update OLED Screen Readout

display.clearDisplay();

// Header Bar

display.setTextSize(1);

display.setCursor(15, 0);

display.println(F("SYSTEM MONITOR"));

display.drawLine(0, 10, 128, 10, SSD1306_WHITE);

// Temperature Readout

display.setCursor(0, 16);

display.print(F("TEMP: "));

display.setTextSize(2);

display.setCursor(45, 14);

display.print(temperature, 1);

display.setTextSize(1);

display.println(F(" C"));

// Humidity Readout

display.setTextSize(1);

display.setCursor(0, 36);

display.print(F("HUMID: "));

display.print(humidity, 1);

display.println(F(" %"));

// Fan Status Bar

display.drawLine(0, 48, 128, 48, SSD1306_WHITE);

display.setCursor(0, 54);

display.print(F("FAN STATUS: "));

display.print(fanON ? F("[ ON ]") : F("[ OFF ]"));

display.display(); // Push buffer to OLED

}